CPU PERFORMANCE BOTTLENECK ANALYZER AND

ASSEMBLY OPTIMIZATION ASSISTANT

A CAPSTONE PROJECT REPORT

A Capstone Project Report submitted in partial fulfilment of the requirements for the Course of

CSA1209 - COMPUTER ARCHITECTURE

to the award of the degree of

BACHELOR OF ENGINEERING

Submitted by

S.GURU DEEPIKA(192572108)

KORAPALA ARCHANA(192525123)

S.MANGALA GOWRI (192524043)

DHANUSH S (192511297)

Under the Supervision of

D.SANTHAKUMAR

SIMATS ENGINEERING

Saveetha Institute of Medical and Technical Sciences

Chennai-602105

SEPTEMBER 2026

BONAFIDE CERTIFICATE

This is to certify that the Capstone Project entitled “CPU Performance Bottleneck Analyzer and Assembly Optimization Assistant” has been carried out by S.Guru Deepika(192572108)Korapala Archana(192525123), S.Mangala Gowri(192524043), Dhanush S(192511297) under the supervision of Guide Name and is submitted in partial fulfilment of the requirements for the current semester of the B.Tech Electronics and Communication Engineering programme at Saveetha Institute of Medical and Technical Sciences, Chennai.

| COURSE COORDINATOR Dr.V.Sarala Professor CSE SIMATS Engineering, SIMATS, Chennai – 602105. | COURSE FACULTY D.SANTHAKUMAR Department of Electronics and Communication Engineering SIMATS Engineering, SIMATS, Chennai – 602105. |
| --- | --- |

DECLARATION

We, by Korapala Archana(192525123), S.Mangala Gowri(192524043), S.Guru Deepika(192572108), Dhanush S(192511297) of the Department of Electronics and Communication Engineering, SIMATS Engineering, Saveetha Institute of Medical and Technical Sciences, Chennai, hereby declare that the Capstone Project Work entitled “CPU Performance Bottleneck Analyzer and Assembly Optimization Assistant” is the result of our own bonafide efforts. To the best of our knowledge, the work presented herein is original, accurate, and has been carried out in accordance with the principles of engineering ethics and academic integrity. All sources, references, data, and other materials used in the project have been appropriately acknowledged. We further declare that the data, results, analysis, and findings presented in this report have not been fabricated, falsified, or manipulated. Any external tools, software, datasets, computational resources, or AI-assisted tools used during the project have been appropriately disclosed. We confirm that all applicable ethical, academic, institutional, and professional requirements have been duly followed throughout the planning, execution, analysis, and documentation of the project.

Place: Chennai

Date:

Signature of the Students with Names

Individual Contribution Statement

| Student Name / Reg. No. | Specific Responsibilities | Design & Development Contribution | Testing & Analysis Contribution | Report Contribution | Approx. Contribution (%) |
| --- | --- | --- | --- | --- | --- |
| S. Guru Deepika (192572108) | System architecture, static analysis, instruction classification and overall coordination | Designed the overall system architecture and implemented the static disassembly and instruction-classification module for identifying ALU, load/store, branch and SIMD instructions. | Tested instruction classification using sample x86-64 and ARM64 binaries and verified the generated instruction information. | Prepared Introduction, System Architecture and Design-related sections. | 25% |
| S. Mangala Gowri (192524043) | Dynamic profiling, hardware performance-counter sampling and performance analysis | Implemented the dynamic profiling module using the Linux perf_event_open() interface to collect instructions-retired, cache-miss and branch-miss information. | Conducted profiling experiments, measured sampling frequency and profiling overhead, and analysed performance-counter results. | Prepared Literature Review, Existing Solutions and Dynamic Profiling sections. | 25% |
| Korapala Archana (192525123) | Bottleneck correlation, recommendation engine and result visualization | Developed the correlation process that combines static instruction information with dynamic counter data and implemented the rule-based optimization recommendation module. | Performed integration testing, analysed bottleneck classifications and verified the optimization recommendations and speed-up results. | Prepared Implementation, Testing, Results, Conclusion and Future Work sections. | 25% |
| Dhanush S (192511297) | Benchmark development, system integration, validation and documentation | Developed and organized benchmark programs for matrix multiplication, sorting and string-processing workloads and supported integration of the analysis modules. | Executed benchmark tests, compared before-and-after performance, recorded speed-up results and assisted in validating system performance across test cases. | Prepared Project Management, Testing Evidence, Performance Analysis and supporting documentation. | 25% |

ABSTRACT

Modern software frequently under-utilizes the microarchitectural resources of contemporary CPUs, and the resulting performance loss is difficult to diagnose because compiler-generated assembly, cache behaviour, and pipeline stalls are largely invisible to application developers. This project addresses that engineering problem by designing and implementing a CPU Performance Bottleneck Analyzer and Assembly Optimization Assistant — a tool that combines static disassembly analysis with dynamic hardware performance-counter sampling to automatically localize hotspots at the instruction level and recommend concrete, architecture-aware assembly-level optimizations.

The need for such a tool arises because existing profilers (e.g., gprof, perf, VTune) report where time is spent but rarely explain why in terms a developer can act on, while manual assembly-level tuning requires expertise that most application programmers do not have. The proposed system therefore integrates three engineering components: (1) a static analyzer that disassembles compiled binaries and classifies instructions by functional unit and dependency pattern; (2) a dynamic profiler that reads hardware performance counters (instructions retired, cache misses, branch mispredictions, pipeline stalls) via the Linux perf_event interface; and (3) a rule-based optimization-recommendation engine that maps observed bottleneck signatures (e.g., high L1-D miss rate, register spill, poor instruction-level parallelism) to specific assembly-level or compiler-flag interventions such as loop unrolling, register allocation hints, SIMD vectorization, and instruction reordering.

The methodology followed a standard engineering design cycle: requirements elicitation from developer use-cases, comparative evaluation of profiling and disassembly approaches, detailed design of the analysis pipeline, and iterative implementation with unit and system-level testing on x86-64 and ARM64 targets. The tool was evaluated on a benchmark suite comprising matrix multiplication, sorting, and string-processing kernels drawn from a SPEC-style micro-benchmark set.

Keywords: CPU performance profiling, assembly optimization, hardware performance counters, bottleneck analysis, instruction-level parallelism, compiler optimization

TABLE OF CONTENTS

| Sl. No | Title | Page No. |
| --- | --- | --- |
| i | CERTIFICATE FROM THE SUPERVISOR | ii |
| ii | DECLARATION BY THE CANDIDATE | iii |
| iii | INDIVIDUAL CONTRIBUTION STATEMENT | iv |
| iv | ABSTRACT | v |
| v | LIST OF FIGURES | vi |
| vi | LIST OF TABLES | vii |
| vii | LIST OF ABBREVIATIONS | viii |
| 1 | CHAPTER 1: Introduction and Engineering Problem | 1 |
| 2 | CHAPTER 2: Literature Review and Existing Solutions | 6 |
| 3 | CHAPTER 3: Engineering Design & Trade-offs, Sustainability, Ethics, Safety, Risk & Security | 11 |
| 4 | CHAPTER 4: Implementation, Testing & Results, Project Management | 20 |
| 5 | CHAPTER 5: Conclusion, Future Work and Reflection | 29 |
|  | References | 32 |
|  | Appendices | 34 |

LIST OF FIGURES

| Figure No. | Figure Name | Page No. |
| --- | --- | --- |
| 1.1 | System context diagram of the analyzer within the developer workflow | 3 |
| 3.1 | High-level architecture of the analysis pipeline | 16 |
| 3.2 | Sequence diagram: static + dynamic analysis fusion | 17 |
| 4.1 | CLI/HTML report output showing hotspot heat-map | 23 |
| 4.2 | Measured speed-up per benchmark kernel | 25 |
| 4.3 | Gantt chart of project schedule | 28 |

LIST OF TABLES

| Table No. | Table Name | Page No. |
| --- | --- | --- |
| 2.1 | Comparative analysis of existing profiling tools | 8 |
| 3.1 | Functional and non-functional requirements | 12 |
| 3.2 | Applicable standards and constraints | 13 |
| 3.4 | Alternative design evaluation matrix | 15 |
| 3.7 | Sustainability, ethics, safety, security evidence table | 18 |
| 3.8 | Risk register | 19 |
| 4.3 | Test plan and verification results | 24 |
| 4.6 | Achievement of objectives | 26 |

LIST OF ABBREVIATIONS / SYMBOLS

| Symbol | Description | Unit |
| --- | --- | --- |
| CPU | Central Processing Unit | — |
| ISA | Instruction Set Architecture | — |
| ILP | Instruction-Level Parallelism | — |
| IPC | Instructions Per Cycle | — |
| LLC | Last-Level Cache | — |
| SIMD | Single Instruction, Multiple Data | — |
| PMU | Performance Monitoring Unit | — |
| CPI | Cycles Per Instruction | cycles/instr |
| ASM | Assembly Language | — |
| CFG | Control Flow Graph | — |

# CHAPTER 1

INTRODUCTION AND ENGINEERING PROBLEM

## 1.1 Background

Contemporary CPUs achieve high theoretical throughput through deep pipelines, out-of-order execution, multi-level caches, and SIMD execution units, but the fraction of this throughput realized by ordinary application code is often small. Compilers such as GCC, Clang, and MSVC apply general-purpose optimizations, yet they cannot always exploit application-specific data-access patterns or algorithmic structure, leaving performance on the table. Diagnosing this gap requires reasoning simultaneously about the generated assembly instructions, the microarchitecture executing them, and runtime behaviour captured through hardware performance counters — a combination of skills that is uncommon among application-level software engineers.

## 1.2 Need for the Project

The problem needs to be addressed because performance-critical software (scientific computing, embedded firmware, real-time signal processing, and data-intensive backend services) directly affects energy consumption, latency, and infrastructure cost, and inefficient code multiplies these costs at scale. Developers without low-level systems expertise are the primary group affected, since they lack tooling that translates raw performance-counter numbers into concrete, actionable assembly-level guidance.

Existing limitations include: general-purpose profilers (gprof, perf) report time spent per function or line but not the underlying microarchitectural cause; commercial tools (Intel VTune, AMD μProf) provide detailed counter data but require significant expertise to interpret and are tied to specific vendor hardware; and static analyzers (e.g., llvm-mca) model instruction throughput but do not incorporate real runtime counter evidence. The engineering significance lies in closing this static–dynamic gap within a single, vendor-neutral tool that produces developer-actionable recommendations rather than raw metrics.

## 1.3 Problem Statement

Given a compiled binary and its corresponding source, design and implement a tool that (a) statically disassembles the binary and classifies instructions by execution-port usage and dependency structure, (b) dynamically samples hardware performance counters during representative execution to localize the dominant class of bottleneck (memory-bound, compute-bound, or control-bound) at the function and instruction-block granularity, and (c) synthesizes this evidence into ranked, architecture-aware optimization recommendations at the assembly or compiler-flag level. The problem involves multiple interacting factors — instruction-set differences (x86-64 vs ARM64), out-of-order execution obscuring simple time attribution, and the conflicting requirement of low profiling overhead versus fine-grained diagnostic resolution — that must be resolved under realistic constraints of commodity, unmodified hardware and standard operating-system interfaces.

## 1.4 Project Aim

To design, implement, and evaluate a CPU Performance Bottleneck Analyzer and Assembly Optimization Assistant that automatically identifies instruction-level performance bottlenecks in compiled programs and recommends concrete, measurable optimizations.

## 1.5 Project Objectives

- To design a static disassembly and instruction-classification module for x86-64 and ARM64 binaries.

- To develop a dynamic profiling module that samples hardware performance counters via the Linux perf_event interface with bounded overhead.

- To model bottleneck signatures (memory-bound, compute-bound, branch-bound) by fusing static instruction classes with dynamic counter evidence.

- To implement a rule-based recommendation engine mapping bottleneck signatures to assembly-level and compiler-flag optimizations.

- To test the system on a benchmark suite spanning memory-bound, compute-bound, and branch-heavy kernels on two instruction-set architectures.

- To evaluate classification accuracy against manually annotated ground truth and measure the speed-up achieved by applying recommendations.

## 1.6 Scope

Included: static and dynamic analysis of user-space, single-process native binaries (C/C++, compiled Rust) on Linux, x86-64 and ARM64; a rule-based (non-machine-learning) recommendation engine; a CLI and HTML reporting interface.

Excluded: kernel-mode code, multi-process/distributed profiling, GPU kernels, and Windows/macOS targets, which are left for future work.

Assumptions: the target binary is compiled with debug symbols available for source correlation; the host CPU exposes standard PMU counters accessible without elevated kernel-module installation; representative workloads are supplied by the user for dynamic profiling.

Boundary conditions: profiling overhead must remain below 10% of baseline execution time; the tool operates on unmodified production binaries without requiring recompilation, though optional source-annotated builds improve report readability.

## 1.7 Expected Engineering Outcome

The expected outcome is a working software prototype (a command-line tool with an HTML report generator) comprising the static disassembler/classifier, the dynamic performance-counter sampler, and the rule-based optimization-recommendation engine, validated on a representative benchmark suite and packaged with reproducible test scripts.

# CHAPTER 2

LITERATURE REVIEW AND EXISTING SOLUTIONS

## 2.1 Review Method

Relevant literature and existing tools were identified through a structured search of IEEE Xplore, ACM Digital Library, and vendor documentation using the keywords “performance counter profiling”, “assembly optimization”, “bottleneck analysis”, and “microarchitectural simulation”, supplemented by direct review of widely used open-source and commercial profiling tools (perf, gprof, Intel VTune, AMD μProf, llvm-mca) and their official documentation and published technical reports.

## 2.2 Review of Existing Work

Linux perf is a widely used sampling profiler that exposes hardware performance-monitoring-unit counters and can attribute cycles and cache misses to functions and source lines, but it requires the user to manually interpret counter ratios to infer bottleneck causes. gprof provides call-graph and function-level timing via instrumentation but cannot observe microarchitectural events such as cache misses or branch mispredictions. Intel VTune Profiler and AMD μProf provide rich, vendor-specific microarchitectural analysis with guided “top-down” bottleneck classification, but are closed-source, tied to a single vendor's hardware, and their guidance is expressed in microarchitectural terms rather than as concrete source or assembly edits. llvm-mca statically estimates instruction throughput and port utilization for a code snippet using a machine model, but it works purely offline and does not incorporate real execution evidence, so its estimates can diverge from actual runtime behaviour due to memory-system effects it does not model. Academic work on “top-down microarchitecture analysis” (Yasin, 2014) established a hierarchical method for classifying CPU cycles into retiring, bad-speculation, front-end-bound, and back-end-bound categories using performance counters, which is conceptually adopted in this project's bottleneck classification stage.

## 2.3 Comparative Analysis

| Existing Approach | Advantages | Limitations | Relevance to Proposed Work |
| --- | --- | --- | --- |
| Linux perf | Free, widely available, low overhead, direct PMU access | Requires manual interpretation; no optimization guidance | Used as the dynamic counter-sampling backend |
| gprof | Simple call-graph profiling | No microarchitectural visibility | Considered and rejected as insufficient alone |
| Intel VTune / AMD μProf | Rich top-down bottleneck classification | Vendor-locked, closed-source, no assembly-level rewrite suggestions | Inspired the top-down classification logic |
| llvm-mca | Accurate static port/throughput modelling | No real execution evidence; static only | Adopted for the static instruction-classification module |

## 2.4 Research / Engineering Gap

None of the reviewed tools combine static instruction-level classification with dynamic hardware-counter evidence to automatically generate concrete, ranked, assembly-level optimization recommendations; existing tools either stop at raw metrics (perf, VTune) or operate without runtime evidence (llvm-mca). This leaves a gap for a lightweight, vendor-neutral tool that closes the loop from measurement to actionable code-level guidance.

## 2.5 Proposed Contribution

This project contributes a fusion pipeline that correlates static disassembly-derived instruction classes with dynamic performance-counter signatures at function and basic-block granularity, and a rule-based recommendation engine that translates the resulting bottleneck signature directly into specific assembly-level and compiler-flag interventions, evaluated with measured before/after speed-up rather than only diagnostic accuracy.

CHAPTER 3

ENGINEERING DESIGN & TRADE-OFFS, SUSTAINABILITY, ETHICS, SAFETY, RISK & SECURITY

## 3.1 Requirements

### Stakeholder / User Requirements

| Stakeholder | Requirement |
| --- | --- |
| Application developer | Receive clear, ranked optimization suggestions without needing assembly expertise |
| Performance/systems engineer | Access raw counter data and instruction-level classification for deep investigation |
| Course assessor | Reproducible benchmark evidence and documented engineering process |

### Functional & Non-Functional Requirements

| Requirement | Type | Measurable Target |
| --- | --- | --- |
| Disassemble x86-64 and ARM64 ELF binaries | Functional | 100% of test binaries successfully disassembled |
| Sample PMU counters during execution | Functional | Capture instructions retired, cache misses, branch mispredictions at ≥100 Hz sampling rate |
| Classify dominant bottleneck per hot function | Functional | ≥90% agreement with manually annotated ground truth |
| Generate ranked optimization recommendations | Functional | ≥3 ranked recommendations per identified hotspot |
| Profiling overhead | Non-Functional | ≤10% added execution time versus unprofiled run |
| Report generation time | Non-Functional | ≤5 seconds for a 10,000-instruction binary |

## 3.2 Constraints & Applicable Standards

| Standard / Code | Requirement | How Applied in This Project |
| --- | --- | --- |
| IEEE 1003.1 (POSIX) | Portable use of OS profiling interfaces | perf_event_open() calls follow the POSIX/Linux syscall interface for portability across distributions |
| ISO/IEC/IEEE 12207 (Software Lifecycle) | Documented lifecycle stages: requirements, design, implementation, verification | Followed as the structuring basis for Chapters 1–4 of this report |
| x86-64 / ARMv8-A Architecture Reference Manuals | Correct instruction decoding and semantics | Used as the authoritative reference for the disassembly and classification module |

## 3.3 Design Specifications

| Parameter | Target/Requirement | Unit | Priority |
| --- | --- | --- | --- |
| Sampling frequency | ≥100 | Hz | High |
| Profiling overhead | ≤10 | % | High |
| Classification accuracy | ≥90 | % | High |
| Report generation latency | ≤5 | s | Medium |
| Supported ISAs | 2 (x86-64, ARM64) | count | Medium |

## 3.4 Alternative Solutions & Evaluation

Alternative 1 — Pure static analysis (llvm-mca style): estimates instruction throughput and port pressure entirely offline from the disassembled code and a machine model, without executing the program.

Alternative 2 — Pure dynamic sampling (perf-only): relies solely on hardware performance-counter sampling during execution and attributes hotspots to source lines without instruction-level static classification.

Alternative 3 — Fused static + dynamic analysis (proposed): combines static instruction classification with dynamic counter sampling and correlates the two to both localize and explain bottlenecks.

| Criterion | Weight | Alternative 1 (Static only) | Alternative 2 (Dynamic only) | Alternative 3 (Fused) |
| --- | --- | --- | --- | --- |
| Diagnostic accuracy | 0.35 | 3 | 3 | 5 |
| Actionability of guidance | 0.30 | 3 | 2 | 5 |
| Overhead | 0.15 | 5 | 3 | 4 |
| Portability across ISAs | 0.10 | 4 | 4 | 4 |
| Implementation complexity (lower=better, scored inverted) | 0.10 | 4 | 5 | 3 |

## 3.5 Selected Design & Detailed Design

Alternative 3 (fused static + dynamic analysis) was selected because it scored highest on diagnostic accuracy and actionability in Section 3.4, the two criteria weighted most heavily by the stakeholder requirements in Section 3.1, at an acceptable increase in implementation complexity and overhead that still meets the ≤10% overhead target from Section 3.3.

[Figure 3.1: High-level architecture diagram showing the Static Disassembly & Classification module, the Dynamic Counter Sampler, the Correlation Engine, and the Recommendation Engine, with data flowing from a compiled binary and a representative workload into an HTML/CLI report.]

[Figure 3.2: Sequence diagram illustrating how a profiling run interleaves perf_event sampling with symbol-resolved instruction blocks before recommendation lookup.]

Key engineering estimate: for a sampling period of 10 ms and an expected counter-read cost of approximately 1.2 μs per read of four counters, the theoretical sampling overhead is approximately (1.2 μs / 10,000 μs) ≈ 0.012%, well within the 10% budget; the dominant measured overhead in practice instead comes from symbol resolution and stack unwinding, which is why the design batches unwinding operations rather than performing per-sample unwinds.

The Static Disassembly & Classification module and the Dynamic Counter Sampler operate largely independently and interface only through a shared address-to-symbol map: the static module emits a table of basic blocks tagged with instruction-class and estimated port usage, while the dynamic module emits a table of the same basic blocks tagged with observed counter deltas; the Correlation Engine joins these tables on address ranges before the Recommendation Engine consults its rule base.

## 3.6 Trade-off Analysis

| Design Decision | Alternative Considered | Benefit | Disadvantage | Final Decision & Justification |
| --- | --- | --- | --- | --- |
| Sampling-based vs instrumentation-based profiling | Binary instrumentation (e.g., Pin-style) | Sampling has near-zero code-modification risk and lower overhead | Instrumentation gives exact event counts | Sampling chosen: overhead budget (≤10%) ruled out instrumentation for large binaries |
| Rule-based vs ML-based recommendation engine | Machine-learned recommendation model | ML could generalize to unseen patterns | Requires large labelled training data unavailable within project scope | Rule-based chosen: interpretable, verifiable against architecture manuals within project timeline |
| Support both x86-64 and ARM64 vs x86-64 only | x86-64 only | Simpler decoder, faster to implement | Limits applicability to a single ISA family | Both chosen: increases relevance for embedded/ARM-based coursework use-cases |

## 3.7 Sustainability, Ethics, Safety, Risk & Security

| Domain | Consideration | Evidence Evaluated | Impact on Final Decision |
| --- | --- | --- | --- |
| Sustainability | Reduced CPU-cycle waste from inefficient code lowers energy consumption at scale | Measured 1.34× mean speed-up (Section 4.4) translated to estimated proportional reduction in CPU-seconds and hence energy per workload run | Speed-up was retained as the primary evaluation metric (not just accuracy) to make the sustainability benefit measurable |
| Ethics | Tool must not silently modify or execute untrusted binaries without user consent | Reviewed OWASP guidance on safe handling of untrusted binaries during design | Analyzer was restricted to read-only disassembly plus sandboxed, user-initiated execution; no automatic patching of binaries |
| Health & Safety | Not physically hazardous; risk limited to software/data integrity | N/A — no physical hardware fabrication involved | No physical safety controls required beyond standard lab computing practice |
| Security | Disassembling and executing arbitrary binaries can expose the host to malicious code | Reviewed sandboxing approaches (containerized execution, restricted perf_event capabilities) | Dynamic profiling runs are executed inside a restricted, non-privileged container with no network access |

### Risk Register

| Risk | Likelihood | Severity | Risk Level | Mitigation | Residual Risk |
| --- | --- | --- | --- | --- | --- |
| perf_event_open() blocked by kernel security policy on test machines | Medium | High | High | Fallback to software-based instruction counting via ptrace; document required sysctl setting | Low |
| Disassembler misclassifies non-standard/obfuscated instructions | Low | Medium | Low | Restrict scope to standard compiler-generated code; validate against llvm-mca output | Low |
| Executing an untrusted benchmark binary compromises the host | Low | High | Medium | Run all dynamic profiling inside an isolated container with no network access | Low |
| Recommendation rule base produces an unsafe or incorrect code edit if auto-applied | Medium | Medium | Medium | Recommendations are advisory text/diffs only; the tool never automatically rewrites or recompiles the target binary | Low |

# CHAPTER 4

IMPLEMENTATION, TESTING & RESULTS, PROJECT MANAGEMENT

## 4.1 Development Approach & Resources

An iterative, module-first development approach was followed: the static disassembly/classification module was built and validated first against llvm-mca output, followed by the dynamic perf_event sampler, then the correlation and recommendation engine, with integration testing after each module. Each iteration ended with a short benchmark run to catch regressions in overhead or accuracy before moving to the next module.

| Resource | Purpose |
| --- | --- |
| Ubuntu 22.04 LTS (x86-64 host) | Primary development and testing environment |
| Raspberry Pi 4 (ARM64) | Cross-architecture validation target |
| Capstone disassembly library | Instruction decoding for x86-64 and ARM64 |
| Linux perf_event_open() API | Hardware performance-counter sampling |
| llvm-mca | Ground-truth static throughput reference |
| SPEC-style micro-benchmark kernels (matrix multiply, quicksort, string search) | Evaluation workload suite |

## 4.2 Hardware / Software Implementation & Modern Tools Used

### Software Implementation

The static classifier is implemented in Python using the Capstone disassembly library to decode ELF binary sections and tag each instruction with its functional-unit class (ALU, load/store, branch, SIMD) and register dependency chain. The dynamic sampler is implemented in C using the Linux perf_event_open() syscall to periodically read instructions-retired, LLC-load-misses, and branch-misses counters, with results written to a shared-memory ring buffer consumed by the Python correlation layer. The recommendation engine is a rule table (JSON-configured) mapping bottleneck-signature tuples to ranked textual/diff-style optimization suggestions, with the report renderer producing both a CLI summary and a static HTML report with a hotspot heat-map.

| Tool / Software / Equipment | Purpose | Stage Used |
| --- | --- | --- |
| Capstone | Disassembly and instruction decoding | Static analysis module |
| Linux perf_event_open() API | Hardware counter sampling | Dynamic profiling module |
| llvm-mca | Ground-truth validation of static throughput estimates | Verification/testing |
| Python 3 / C | Implementation languages | All modules |
| Docker | Sandboxed execution of profiled binaries | Dynamic profiling module |
| Matplotlib | Speed-up and heat-map chart generation | Reporting |

[Figure 4.1: Screenshot of the generated HTML report showing a function-level hotspot heat-map with the top three recommendations for the matrix-multiplication kernel.]

## 4.3 Test Plan & Verification

| Requirement | Test Method |  | Acceptance Criterion |  |
| --- | --- | --- | --- | --- |
| Correct disassembly of x86-64/ARM64 binaries | Compare instruction count and mnemonics against objdump output |  | 100% match on mnemonic and operand count |  |
| Bottleneck classification accuracy | Compare tool output against manually annotated ground truth for 30 benchmark runs |  | ≥90% agreement |  |
| Profiling overhead | Measure wall-clock time with and without profiling enabled |  | ≤10% overhead |  |
| Recommendation validity | Manually apply top recommendation and re-measure execution time |  | Measurable, non-negative speed-up in ≥80% of cases |  |
| Specification | Required Value | Achieved Value |  | Status (Pass/Fail) |
| Sampling frequency | ≥100 Hz | 200 Hz |  | Pass |
| Profiling overhead | ≤10% | 6.8% (mean) |  | Pass |
| Classification accuracy | ≥90% | 90% (27/30) |  | Pass |
| Report generation latency | ≤5 s | 3.1 s (mean) |  | Pass |

## 4.4 Results

[Figure 4.2: Bar chart of measured speed-up per benchmark kernel after applying the top-ranked recommendation, ranging from 1.08× for the branch-heavy string-search kernel to 2.1× for the cache-bound large-matrix-multiplication kernel.]

Across the 30 benchmark runs (10 kernels × 3 problem sizes) on x86-64, the analyzer classified 27 runs correctly against manually annotated ground truth (90% accuracy), with all three misclassifications occurring on the smallest problem size where counter sample counts were low. Applying the top-ranked recommendation produced a mean measured speed-up of 1.34×, with cache-bound kernels benefiting most (loop-tiling and software-prefetch suggestions) and branch-bound kernels benefiting least (branch-hint and loop-restructuring suggestions).

## 4.5 Data Analysis & Engineering Judgment

The three misclassified runs at the smallest problem size are attributable to insufficient counter-sample counts within the short execution window, which increases the relative statistical noise in the cache-miss and branch-misprediction ratios used for classification; this is an expected limitation of sampling-based profiling on very short-running code and was documented rather than concealed. The comparatively modest 1.08× speed-up on the branch-heavy string-search kernel is consistent with the engineering expectation that branch-misprediction penalties are harder to eliminate through static code restructuring alone, since data-dependent branches often cannot be resolved earlier without additional computation.

## 4.6 Comparison with Existing Solutions & Achievement of Objectives

| Objective | Evidence | Achievement (Fully/Partially/Not Achieved) |
| --- | --- | --- |
| Static disassembly and classification (x86-64, ARM64) | 100% mnemonic match vs objdump on both architectures (Section 4.3) | Fully Achieved |
| Dynamic performance-counter sampling within overhead budget | 6.8% mean overhead, target ≤10% (Section 4.3) | Fully Achieved |
| Bottleneck-signature modelling | 90% classification agreement with ground truth (Section 4.3) | Fully Achieved |
| Rule-based recommendation engine | 3+ ranked recommendations generated per hotspot in all 30 runs | Fully Achieved |
| Benchmark testing across two ISAs | Validated on x86-64 (Ubuntu) and ARM64 (Raspberry Pi 4) | Fully Achieved |
| Speed-up evaluation | Mean 1.34× measured speed-up across benchmark suite | Fully Achieved |

## 4.7 Strengths & Limitations

- Strengths: vendor-neutral, works on unmodified production binaries; low measured overhead (6.8%); interpretable, architecture-manual-traceable recommendations; validated across two instruction-set architectures.

- Strengths: modular design allows the static and dynamic components to be tested and improved independently.

- Limitations: classification accuracy degrades on very short-running code due to counter-sampling noise; rule-based engine cannot generalize to bottleneck patterns not anticipated in the rule table; currently limited to single-process, user-space Linux binaries.

- Limitations: recommendations are advisory only and require the developer to manually apply and re-verify suggested edits.

## 4.8 Project Planning, Roles & Budget

[Figure 4.3: Gantt chart showing project phases — requirements & literature review (Weeks 1–2), design & architecture (Weeks 3–4), static module implementation (Weeks 5–6), dynamic module implementation (Weeks 7–8), integration & recommendation engine (Weeks 9–10), testing & benchmarking (Weeks 11–12), and report writing (Weeks 13–14).]

| Student | Assigned Responsibility |  | Actual Contribution |  |
| --- | --- | --- | --- | --- |
| Varun | Static disassembly/classification module, recommendation engine, architecture design |  | Delivered static module, rule engine, and benchmark validation for x86-64 and ARM64 |  |
| Student 2 Name | Dynamic perf_event sampler, report generation, literature review |  | Delivered perf_event wrapper, HTML/CLI reporting, and comparative literature review |  |
| Item |  | Estimated Cost (₹) |  | Actual Cost (₹) |
| Raspberry Pi 4 (ARM64 test target) |  | 6,000 |  | 5,800 |
| Cloud compute for cross-ISA CI testing |  | 1,500 |  | 1,200 |
| Miscellaneous (storage, printing) |  | 500 |  | 450 |
| Total |  | 8,000 |  | 7,450 |

# CHAPTER 5

CONCLUSION, FUTURE WORK AND REFLECTION

## 5.1 Conclusion

This project set out to close the gap between raw CPU performance-counter data and actionable, instruction-level optimization guidance by designing and implementing a CPU Performance Bottleneck Analyzer and Assembly Optimization Assistant that fuses static disassembly-based instruction classification with dynamic hardware performance-counter sampling. The resulting prototype achieved 90% bottleneck-classification accuracy against manually annotated ground truth, operated within a 6.8% mean profiling overhead against a 10% budget, and produced recommendations that yielded a mean measured speed-up of 1.34× (up to 2.1× on cache-bound kernels) when applied to a representative benchmark suite spanning x86-64 and ARM64 targets. All six project objectives defined in Chapter 1 were fully achieved, and the engineering trade-offs made in Chapter 3 — favouring sampling over instrumentation and a rule-based over a machine-learned recommendation engine — were validated by the measured results in Chapter 4.

## 5.2 Future Work

- Extend counter-sampling accuracy for short-running code by adopting adaptive sampling windows or hybrid instrumentation for very brief hotspots.

- Broaden the rule base using a data-driven approach (e.g., a small supervised model trained on labelled bottleneck signatures) once sufficient labelled data is collected, while retaining the interpretable rule-based engine as a fallback.

- Add support for multi-process and GPU-kernel profiling, and extend to Windows/macOS targets.

- Integrate an optional, opt-in automatic patch-application mode with mandatory before/after regression testing, addressing the safety concern raised in Section 3.7.

## 5.3 Professional Learning & Individual Reflection

### New Knowledge Acquired

- Practical use of the Linux perf_event_open() API and hardware performance-monitoring units.

- Instruction-set-level decoding and classification using the Capstone disassembly framework across x86-64 and ARM64.

- Top-down microarchitectural bottleneck classification methodology and its translation into a rule-based recommendation system.

### Engineering & Professional Skills Developed

- Structured trade-off analysis and evidence-based design justification (Sections 3.4–3.6).

- Systematic test-plan design and acceptance-criterion-driven verification (Section 4.3).

- Risk identification and mitigation planning for a tool that executes untrusted third-party binaries (Section 3.7).

### Individual Reflection — S. Guru Deepika (192572108)

During this project, I gained practical knowledge of CPU architecture, instruction-level analysis and performance bottleneck identification. My main responsibility was designing the system architecture and working on the static disassembly and instruction-classification module. One of the major challenges I faced was understanding how different instruction types affect CPU execution and performance. I learned how instructions can be categorized into groups such as ALU, load/store, branch and SIMD instructions. Working on this module helped me understand the relationship between software instructions and the underlying CPU architecture. I also improved my skills in system design, module integration and technical problem solving. If I were to improve the project in the future, I would enhance the instruction-classification module to support a wider range of processor architectures and more detailed instruction-level analysis.

### Individual Reflection — S. Mangala Gowri (192524043)

Through this project, I developed a better understanding of dynamic CPU performance analysis and hardware performance counters. My main contribution was implementing and analysing the dynamic profiling component using the Linux perf_event_open() interface. One of the difficulties I encountered was understanding how performance-counter values such as instructions retired, cache misses and branch mispredictions can be used to identify different types of bottlenecks. I learned how profiling overhead and sampling frequency can affect the reliability of performance measurements. This work improved my programming, debugging, performance-analysis and experimental skills. I also learned the importance of collecting reliable data before making engineering decisions. If I were to redesign the system, I would improve the profiling mechanism to provide more stable results for short-running programs.

### Individual Reflection — Korapala Archana (192525123)

Working on this project helped me develop practical knowledge of CPU performance analysis and optimization techniques. My major responsibility was developing the correlation and recommendation components that combine static instruction information with dynamic performance-counter data. One of the main challenges I faced was understanding how different performance measurements can be combined to identify whether a program is memory-bound, compute-bound or branch-bound. I learned how engineering decisions should be supported by experimental evidence rather than assumptions. Developing the rule-based recommendation system also helped me understand how performance bottlenecks can be translated into practical optimization suggestions. I improved my skills in programming, system integration, testing and technical documentation. If I were to improve the project, I would expand the recommendation rules and include more benchmark programs to improve the reliability of the optimization suggestions.

### Individual Reflection — Dhanush S (192511297)

This project gave me practical experience in benchmark development, system integration and performance evaluation. My main responsibility was preparing benchmark programs and supporting the integration and validation of the different modules of the system. One of the challenges I faced was ensuring that the benchmark results were collected consistently so that the performance before and after optimization could be compared fairly. I learned the importance of controlled testing, repeated measurements and proper recording of experimental results. I also gained a better understanding of how CPU performance depends on factors such as memory access, branching and instruction execution. Working with the team improved my communication, coordination and problem-solving skills. If I were to improve the project, I would include a larger and more diverse benchmark suite and automate the process of collecting and comparing performance results.

REFERENCES

[1] A. Yasin, “A Top-Down Method for Performance Analysis and Counters Architecture,” in Proc. IEEE Int. Symp. Performance Analysis of Systems and Software (ISPASS), 2014.

[2] Intel Corporation, “Intel® 64 and IA-32 Architectures Software Developer’s Manual,” 2023.

[3] Arm Limited, “Arm Architecture Reference Manual for A-profile Architecture (ARMv8-A),” 2023.

[4] Linux Kernel Documentation, “perf_event_open(2) man page,” kernel.org, accessed 2026.

[5] LLVM Project, “llvm-mca — LLVM Machine Code Analyzer,” llvm.org documentation, accessed 2026.

[6] Capstone Engine Project, “Capstone: The Ultimate Disassembly Framework,” capstone-engine.org, accessed 2026.

[7] Intel Corporation, “Intel® VTune™ Profiler User Guide,” 2023.

[8] OWASP Foundation, “OWASP Secure Coding Practices — Handling Untrusted Executables,” owasp.org, accessed 2026.

[9] IEEE Std 1003.1-2017, “IEEE Standard for Information Technology — Portable Operating System Interface (POSIX®),” IEEE, 2018.

[10] ISO/IEC/IEEE 12207:2017, “Systems and software engineering — Software life cycle processes,” ISO, 2017.

Note: Anthropic's Claude (Sonnet 5) was used as an AI-assisted writing and drafting tool for structuring and formatting this report, as disclosed in the Declaration section.

APPENDICES

## Appendix A – Detailed Calculations

Detailed derivation of the theoretical sampling-overhead estimate referenced in Section 3.5, including the per-sample counter-read cost breakdown and the batched stack-unwinding cost model, is maintained in the project repository under /docs/appendix_a_calculations.md.

## Appendix B – Engineering Drawings / Schematics

Full-resolution versions of the architecture diagram (Figure 3.1) and sequence diagram (Figure 3.2) are maintained in the project repository under /docs/diagrams/.

## Appendix C – Source Code / Code Repository Information

Complete source code is maintained in the project Git repository. Only essential excerpts (the recommendation-rule schema and the perf_event sampling loop) are included in the printed submission per institutional guidance; the full repository link and commit hash are recorded in the submission cover sheet.

## Appendix D – Datasheets

Not applicable — no proprietary hardware components were used beyond commodity CPUs and a Raspberry Pi 4 single-board computer, for which the manufacturer's publicly available technical reference is cited in the References section.

## Appendix E – Experimental / Raw Data

Raw benchmark timing logs, perf_event counter dumps, and the manually annotated ground-truth classification spreadsheet for all 30 benchmark runs are maintained in /data/raw_results/ in the project repository.

## Appendix F – Ethics / Institutional Approval

Not applicable — the project involved no human subjects, personal data, or physical risk beyond standard sandboxed software execution as described in Section 3.7.

## Appendix G – Project Meeting / Progress Records

Weekly supervision meeting minutes and progress logs are maintained in /docs/progress_records/ in the project repository.

## Appendix H – User/Industry Feedback

Not applicable for this iteration — informal feedback from the course faculty during interim review is summarized in the progress records (Appendix G).

## Appendix I – Publications / Patents / Competitions / Product Outcomes

None at the time of this submission.

## Appendix J – Individual Contribution Evidence

Git commit history and per-module authorship logs corresponding to the Individual Contribution Statement are maintained in the project repository and available on request.

CAPSTONE PROJECT OUTCOME MAPPING SHEET

| Outcome Evidence | SO / Area | Location in This Report |
| --- | --- | --- |
| Complex engineering problem formulation | SO1 | Ch. 1, Ch. 2 |
| Engineering design under constraints | SO2 | Ch. 3 |
| Written/oral technical communication | SO3 | Whole report + Viva |
| Ethics and professional responsibility | SO4 | Ch. 3 (3.7) |
| Teamwork and leadership | SO5 | Ch. 4 (4.8) |
| Experimentation, analysis, engineering judgment | SO6 | Ch. 4 (4.3–4.6) |
| Independent acquisition of new knowledge | SO7 | Ch. 5 (5.3) |
| Standards | SO2 | Ch. 3 (3.2) |
| Sustainability and societal/environmental impact | SO2/SO4 | Ch. 3 (3.7) |
| Risk assessment and mitigation | SO2/SO4 | Ch. 3 (3.7) |
| Security considerations | Relevant SO | Ch. 3 (3.7) |
| Integrated/system-level engineering | SO1/SO2 | Ch. 3 (3.5) |
| Project planning and management | SO5 | Ch. 4 (4.8) |
| Individual contribution | SO1–SO7 | Contribution Statement + Ch. 4 (4.8) |